Τελικός Προγραμματισμός για RobIn

#include <Servo.h>

Servo myservo;     // Πρώτος  servo

Servo myservo1;   // Δεύτερος servo

int i=0;              // μεταβλητή για να κάνει βήματα πίσω

int pos = 0;          // γωνίες πρώτου servo

int pos1 = 0;        // γωνίες δεύτερου servo

int tonePin = 4;    //Piezo

int trig=7;         // Sonar

int echo=8;         // Sonar

int LED=6;          // LED

int duration, distance;

void setup() {

myservo.attach(9);     //  η εντολή του πρώτου servo πάει στο pin 9

myservo1.attach(10);   //  η εντολή του δεύτερου servo πάει στο pin 10

Serial.begin(9600);

pinMode (trig, OUTPUT);

pinMode (echo, INPUT);

pinMode (LED, OUTPUT);

}

void loop() {

// digitalWrite (trig, LOW);

// delayMicroseconds (2);

digitalWrite (trig, HIGH);

delayMicroseconds (10);

digitalWrite (trig, LOW);

duration= pulseIn (echo, HIGH);

distance= duration / 58.138;

analogWrite (LED ,distance);

distance=map(distance, 0, 45, 0, 45);

if (distance <= 0){                       // Ορίζονται η αποστάσεις που "φωνάζει" το Piezo

distance=0;

}

else if (distance >= 5 && distance <= 14){

tone(tonePin, 200000, 200);

delay(100);

for (i=1;i<10;i=i+1){

delay(100);

for (pos1 = 80; pos1 <= 120; pos1 += 1) { //  mesaia podia

// in steps of 1 degree

myservo1.write(pos1);                     // tell servo to go to position in variable 'pos'

delay(3);                                 // waits 3ms for the servo to reach the position

}

for (pos = 50; pos <= 120; pos += 1) {      // mpros-piso podia

// in steps of 1 degree

myservo.write(pos);                       // tell servo to go to position in variable 'pos'

delay(3);                                 // waits 3ms for the servo to reach the position

}

for (pos1= 120; pos1 >= 80; pos1 -= 1) {    //  mesaia podia

myservo1.write(pos1);                     // tell servo to go to position in variable 'pos'

delay(3);                                 // waits 3ms for the servo to reach the position

}

for (pos = 120; pos >= 50; pos -= 1) {      // mpros-piso podia

myservo.write(pos);                       // tell servo to go to position in variable 'pos'

delay(3);                                 // waits 3ms for the servo to reach the position

}

}

}

else if (distance >= 15 && distance <= 24){

tone(tonePin,5000, 200);

}

else if (distance >= 25 && distance <= 35){

tone(tonePin, 1000, 200);

}

// Serial.println(distance);

delay(200);

noTone(tonePin);

for (pos = 50; pos <= 120; pos += 1) {      //  mpros-piso podia

// in steps of 1 degree

myservo.write(pos);                       // tell servo to go to position in variable 'pos'

delay(3);                                 // waits 3ms for the servo to reach the position

}

for (pos1 = 80; pos1 <= 120; pos1 += 1) { //  mesaia podia

// in steps of 1 degree

myservo1.write(pos1);                     // tell servo to go to position in variable 'pos'

delay(3);                                 // waits 3ms for the servo to reach the position

}

for (pos = 120; pos >= 50; pos -= 1) {      //  mpros-piso podia

myservo.write(pos);                       // tell servo to go to position in variable 'pos'

delay(3);                                 // waits 3ms for the servo to reach the position

}

for (pos1= 120; pos1 >= 80; pos1 -= 1) {    //  mesaia podia

myservo1.write(pos1);                     // tell servo to go to position in variable 'pos'

delay(3);                                 // waits 3ms for the servo to reach the position

}

}