// C++ code

//

int Apostatsi = 0;

long readUltrasonicDistance(int triggerPin, int echoPin)

{

pinMode(triggerPin, OUTPUT);  // Clear the trigger

digitalWrite(triggerPin, LOW);

delayMicroseconds(2);

// Sets the trigger pin to HIGH state for 10 microseconds

digitalWrite(triggerPin, HIGH);

delayMicroseconds(10);

digitalWrite(triggerPin, LOW);

pinMode(echoPin, INPUT);

// Reads the echo pin, and returns the sound wave travel time in microseconds

return pulseIn(echoPin, HIGH);

}

void setup()

{

pinMode(11, OUTPUT);

pinMode(12, OUTPUT);

pinMode(13, OUTPUT);

}

void loop()

{

Apostatsi = 0.01723 * readUltrasonicDistance(7, 6);

if (Apostatsi > 100) {

digitalWrite(11, LOW);

digitalWrite(12, LOW);

digitalWrite(13, LOW);

} else {

if (Apostatsi > 50) {

digitalWrite(11, HIGH);

digitalWrite(12, LOW);

digitalWrite(13, LOW);

} else {

if (Apostatsi > 25) {

digitalWrite(11, HIGH);

digitalWrite(12, HIGH);

digitalWrite(13, LOW);

} else {

digitalWrite(11, HIGH);

digitalWrite(12, HIGH);

digitalWrite(13, HIGH);

}

}

}

delay(10); // Delay a little bit to improve simulation performance

}